# Automatic-filter type 6.18

Filtration and the fully automatic backflushing are based on the bipolar functional principle.

The contaminated fluid enters from both sides into the inside of the candle.

By this, the the particles of contamination are retained over the whole length of the candle.

So, the entire filtration area is uniformly used.

The differential or time depending regeneration of the filter is performed without interruption of operation.

The two-sided open candle and a rotating cover-arm (for dynamic throttling), effectuate a axial– and cross-backflushing to the point and distribution of the backflush energy on the entire lenght of the filter-candle.

The automatic filter is a ready to work filter unit.

The mud drain valve can optional actuated pneumatically or hydraullically.

The electronic control box belongs to the standard equipment.

Type 6.18 automatic filter:

Size / nominal diameter:			DN 50 – DN 900  ( 2“ – 36“ )

Housing material:				cast iron, welded steel or stainless steel

Internal parts:					stainless steel

Kind of filter-candle:				two-sided open cylindrical candle

Type of filter-candle:				Cross-, longitudinal wedgewire or mesh-candle

Candle quantity:				...... pcs.

Filter area:					...... cm²

Operational data:

Medium:					...... – water

Flow rate:					...... – m³/h

Allowable operation – pressure:		0 – 16 bar  (higher operation-pressure on request)

Allowable operation – temperature:		0 – 60°C

Grade of filtration:				50 micron and larger

Backflush – medium:				Clean medium itself

Power supply:					400V, 60 Hz

Manufacturer: 				Boll & Kirch Filterbau GmbH

Type:						Selfclean 6.18